J/105 CLASS ASSOCIATION RULES

Modified for use at Deauville week June 2006

May 11, 2006

OBJECTIVES

1.1 These rules are (a) to preserve J/105's recreational features, including ease of handling, low cost of ownership, safety, comfort, and equality of performance while maximizing participation at J/105 events, and (b) to foster one-design racing in J/105s among predominately amateur crews.

1.2 Except where variations are specifically permitted by these rules, J/105s shall be alike in hull, deck, keel, rudder and spar construction, weight and weight distribution, sail plan, equipment, and interior furnishings.

1.3 All yachts, competing in one design or class sponsored events, shall comply with standard specifications published by J Boats, Inc. and these class rules. No alterations or modifications are permitted unless explicitly permitted by these rules.

ADMINISTRATION

2.1 The J/105 Class Association shall be the sole authority worldwide for the conduct and management of J/105 Class events. The management of class events and class rules shall be conducted with the involvement of the copyright holder and Class Executive Committee, as defined in the J/105 Class Constitution.

2.3 The ExCom shall keep members informed of proposed rule changes and class activities.

2.4 J/105s shall be built only by builders licensed to do so by J Boats, Inc., and shall comply with J/105 building specifications detailed by the copyright holder.

2.5 No boat shall be deemed a J/105 class boat until it has been completed with a building number assigned by J Boats, Inc. moulded into the transom and equipped to qualify for one-design class racing.

2.6 Moulded fibreglass parts, spars, rudder, keel, engine and prop shall be supplied by J Boats, Inc. or their authorized J/105 builders.

2.7 The official language for the class shall be English. The word "shall" is mandatory. The word "may" is permissive.

2.8 Advertising for the Class shall comply with the International Racing Federation’s Racing Rules of Sailing, as in effect from time to time (referred to herein as "RRS") and Category C of Appendix 1 thereto (or any successor appendix).

MEMBERSHIP AND ELIGIBILITY

3.1 An Active Member shall be an Owner of a J/105 that complies with Class Rules and whose annual dues have been paid. Subject to Section 3.5, Group 1, 2 or 3 status is to be determined in accordance with the ISAF Eligibility Code.

3.2 An Associate Member is a family member, crew or other individual supporting the Class Association by paying annual dues.

3.3 An Owner is a person who legally owns 100% of the yacht or is a Group 1 competitor who is an at least one-third partner in terms of financial investment in the purchase of the complete yacht and the cost of its operations. All Owners shall be Active or Associate Members of the J/105 Class Association, provided there shall be only one Active Member per boat. Long term leasing is considered as ownership

3.4 The Driver shall either be an Active Member or an Associate Member. Any Driver who is an Associate Member shall be a Group 1 competitor and be either (a) a member of an Owner's immediate family or (b) a long term shipmate and friend of an Owner. A Driver is defined as the person or persons who, excepting for momentary absence due to personal or shipboard needs, steer(s) the yacht during the 5 minutes prior to and including the start, until the finish.

3.4.1  Any Driver of a boat who is not a Group 1 competitor must be either (a) the 100% Owner of such boat or (b) the Charterer of the boat and the Owner is not on board.

3.5  Reserved for future use

3.6 No crew aboard may be compensated in any way or receive financial benefit for racing in a J/105 sanctioned event. However, a crew may occasionally accept reimbursement for reasonable out-or-pocket expenses of travel, living  accommodations and meals necessary for participation in an event.

MEASUREMENT and Advertising

4.1 A measurer shall not measure a yacht in which he is an interested party.

4.2 Active Members and skippers have the responsibility of ensuring that their yacht complies with these Class Rules.

4.3 By participating in J/105 events or for J/105 one-design trophies, owners agree to permit J/105 Class Officers or Race Committee or their appointed representatives to board their boat unaccompanied for the purpose of inspecting, measuring or equalizing at any time during a regatta or within 24 hours prior before or after.

4.4 Advertising is only allowed on the boom, fore third of hull side, aft third of hull side and on the transom.

EQUIPMENT RULES

5.1 Standard factory supplied furnishings and equipment, including but not limited to the swim ladder, jib roller furling, emergency tiller (wheel boats only) 12 volt battery, igloo cooler, holding tank, head, panelling under and on top of the V-berth, floor boards, main settee bunk cushions, and bulkhead door shall not be relocated or removed when racing. Exception: forward V-berth cushions, dodger, shackles for main halyard and outhaul, block at end of bow sprit and snuffer cleats on the starboard side of the cabin may be removed.

5.2 Safety equipment shall, at least, conform to ORC regulations prescribed by the applicable Notice of Race and/or Sailing Instructions.

5.3 PERMITTED while racing:

5.3.1 Tactical instrumentation.

5.3.2 Additional, non-electronic and or electronic compasses.

5.3.3 GPS/Loran/VHF/Radar.

5.3.4 Interior cruising and day-sailing amenities which don't enhance performance.

5.3.5 Foredeck lifeline netting, shroud rollers & other anti-chafing gear.

5.3.6 A fine tune system for the mainsheet with a maximum 24:1 power ratio may be installed over the coarse tune block or on a bracket mounted on the opposite side of the traveller at the same height as the traveller.

5.3.7 Installed genoa tracks not used.

5.3.8 Effective until January 1, 2006: U-bolts or pad eyes outboard of standard jib track for barberhauling the jib, or affixing blocks to the stanchion bases or chain plates for the same purpose.

Effective on January 1, 2006 and thereafter: U-bolts or pad eyes and blocks attached to them, in each case outboard of standard jib track for barber-hauling the jib outboard, or blocks affixed to the stanchion bases or chain plates for the same purpose, provided that the purchase of any barber hauler shall not exceed 2:1; trimming the lazy windward jib sheet, but no additional equipment or running rigging for barberhauling the jib inboard is permitted.

5.3.9 Spinnaker sheet twings led to stanchion bases or midships pad eyes, with additional cam cleats.

5.3.10 Trimming the lazy windward jib sheet, or running a barberhauler through the cabin handrail to barberhaul the jib lead inboard.

5.3.11 Block and tackle cunningham to ring at top of Quickvang.

5.3.12 Moving mainsheet swivel base forward of traveler.

5.3.13 Driver foot braces.

5.3.14 Non-factory tiller that when weighed with standard tiller head, tiller straps and fasteners is a minimum of 10.9 pounds.

5.3.15 Toggles in the backstay or forestay, or both.

5.3.16 Mast head spinnaker halyard or halyards

5.4 NOT PERMITTED while racing:

5.4.1 Halyard Locks or Hooks.

5.4.2 Holes or Tubes which feed halyards or control lines through the deck, hull or transom.

5.4.3 Altering Rudder or Keel Profile or exceeding tolerances in Official Offsets.

5.4.4 Spinnaker sheets of less than 10mm diameter.

5.4.5 Use of a mast, boom or bowsprit which has been modified in any way, such as cutting off the mast butt to increase rake.

SAILS

6.1 A sail shall comply with the class rules in effect on the date of delivery of the sail (rule 6.9) or at event measurement.

6.2 All sail measurements shall be carried out in accordance with the Equipment Rules of Sailing ("ERS").

Terms used in these class rules in bold letters are used as defined in the ERS.

6.3 Sails used during a class event shall be limited to one mainsail, two jibs and two spinnakers;

6.4 MAINSAIL.

6.4.1 The mainsail shall be manufactured from any fabric allowed under IRC

6.4.2 The dimensions of the mainsail shall not exceed: luff length – 12,650 mm; foot length – 4,450 mm; leech length – 13,260 mm; half width – 2,890 mm; three-quarter width – 1,685 mm; and top width – 178 mm.

6.4.3 Four battens of any length shall be fitted so that the centre of the batten pockets shall divide the leech of the mainsail into five equal parts with a tolerance +/- 80 mm.

6.4.4 The tack ring of the mainsail shall be affixed in the standard tack fitting and the clew of the mainsail shall not be allowed to float free from the boom. The foot may otherwise be loose. At least one serviceable row of reef points shall be installed approximately parallel to the foot, with the tack point and the centre of the reef point in the luff to be no closer than 1830mm. The mainsail shall be attached to the mast with sail slides. Windows are permitted.

6.4.5 The class insignia in blue with overall dimensions approximately 450x900mm shall be affixed on both sides of the sail between the upper two batten pockets, with the bar underneath the “J” located on, and nearly perpendicular to, a line through the mid foot point and a point on the top of the sail equidistant from head point and aft head point. National letters and sail numbers shall conform with the RRS and shall be placed between the middle two batten pockets.

6.5 JIB.

6.5.1 Effective until September 1, 2005: The roller jib shall be constructed of any fabric allowed under IRC

6.5.2 The dimensions of the jib shall not exceed: luff length – 12,150 mm; leech length 11,280 mm;

foot length – 4,450 mm; half-width – 2,020 mm; and top width – 80 mm.

6.5.3 The jib shall have a minimum weight of 23.0 lbs. (10.35 kg), including any battens.

6.5.4 Not more than 3 battens of any length are permitted on the leech, provided that they do not restrict rolling the jib on the furler. Windows are permitted.

6.5.5. The jib shall have a hollow leech and while racing, shall be attached to and operated on the standard roller furling system using #6 luff tape. UV protective tape may be applied to the leech and foot to cover the sail when roller furled.

6.6 SPINNAKER.

6.6.1 The spinnaker shall be manufactured from woven ply consisting of nylon. The ply weight of the body of the sail shall be not less than 44 g/m2.

6.6.2 The area of the sail ("SA") shall not exceed 95m2 based on the following formula:

SA = [(luff length + leech length) * .25 * foot length] + [(half width – .5 * foot length) * (leech length + luff length)] ÷ 3

where luff length shall not be greater than 15,100 mm nor less than 13,600 mm,

leech length shall not be greater than 12,140 mm and half width shall not be less than .65 * foot length.

6.6.3 Adjustable leech, luff and foot lines may be fitted.

6.7 Heavy weather jibs of less than 80% LP, a storm trysail or 2nd mainsail reef at least 40% up from the tack required by ORC regulations are additionally permitted,

Additional rules

7.1 The maximum crew number including the helmsman shall be 6

7.2 When not in the process of setting, flying or taking down the spinnaker, the bowsprit shall be retracted so that the tip of the sprit is aft of the forward most point on the bow. Approaching a windward mark without the spinnaker set, the bowsprit shall not be extended until the bow of the boat has passed the mark. The bowsprit shall be retracted at the first reasonable opportunity after taking the spinnaker down.

7.4 To control mast rake, the headstay system length, measured between the centreline of the headstay pin on the mast to the intersection of the stem line and the sheer line at the bow, shall not be greater than 13035mm nor less than 12985mm.

7.5 The torso of the driver, while steering in a race, must at all times remain aft of the mainsheet traveller.

7.6 Notwithstanding the provisions of RRS 42, 42.3 or 49, hanging on the mast or shrouds to promote roll tacking or roll gybing is prohibited at all times. For purposes of RRS 49.2, roll tacking or roll gybing shall not be a "necessary task."

7.7 A yacht that that has been protested for infringing rule 7.2, 7.5, 7.6 or 7.9 may exonerate herself by promptly making one complete 360 degree turn, including one tack and one gybe unless the boat caused serious damage or gained a significant advantage in a race or series.

7.8 Unless otherwise specified in the sailing instructions for a regatta, all rulings of the Technical Committee shall be incorporated herein by reference and shall constitute rules for purposes of the RRS.

7.9 While racing, the standing rigging (other than the back stay) and the location of the mast butt shall not be adjusted